# 1. Reformulated Core Research Questions (Structured)

## A. Immediate Hydrodynamic Response (Rudder–Propeller Interaction)

- What is the standard maneuvering doctrine for rudder and propulsion use in an immediate-sight MOB scenario?

- How does hard-over rudder toward the casualty side influence:

  - pivot point shift

  - stern lateral displacement

  - propeller trajectory relative to the casualty

- Under what conditions should:

  - propulsion be maintained (to enhance turning moment), vs.

  - propulsion be reduced or set to neutral (to mitigate propeller injury risk)?

## B. “Kick Effect” Quantification (Critical Gap in Practice)

- What is the time scale (seconds) required for the initial stern swing (“kick effect”) after hard-over rudder application?

- What is the minimum heading deviation (Δψ) required to ensure:

  - the propeller disc is no longer intersecting the casualty’s drift path?

- Can this be formalized as:

  - a function of vessel parameters (Lpp, speed, rudder angle, advance/transfer)?

## C. Propeller Clearance Criteria (Safety Threshold Definition)

- When can a casualty be considered hydrodynamically and geometrically clear of the propeller?

Evaluate based on:

- Heading change criterion (e.g., 30°, 45°, 60°?)

- Lateral stern displacement (transfer distance)

- Wake field decay / propeller wash dissipation

- Relative drift (wind/current vs vessel motion)

## D. Rudder Application Limits (Operational Envelope)

- What is the optimal rudder strategy for immediate casualty protection:

  - Maximum rudder (≈35°) vs. controlled rudder?

- How long should hard-over rudder be maintained:

  - time-based (seconds), or

  - geometry-based (heading change / position)?

- Is there a standard seamanship threshold, or is it:

  - vessel-specific (turning circle characteristics)?

## E. Separation Phase vs Recovery Phase

- Distinguish clearly:

  - Phase 1: Immediate Protection (first 10–30 seconds)

  - Phase 2: Recovery Maneuver (Anderson / Williamson / Scharnow)

- What are the decision boundaries between:

  - “protect from propeller”

  - “initiate recovery maneuver”

# 2. Hydrodynamic Interpretation (What Actually Happens)

## Pivot Point & Stern Motion

- When moving ahead:

  - Pivot point ≈ 0.2–0.3 Lpp from bow

- Applying hard rudder:

  - Bow turns toward rudder direction

  - Stern swings opposite direction

Implication for MOB:

- If casualty falls starboard → rudder hard starboard

→ stern swings to port → propeller moves away

## Time Scale of “Kick Effect”

From maneuvering dynamics:

- Rudder force builds over:

  - ~3–10 seconds (flow establishment over rudder)

- Noticeable stern swing:

  - begins almost immediately (~2–5 sec)

- Effective lateral displacement:

  - achieved within first 10–20 seconds

This depends on:

- speed (V)

- rudder angle (δ)

- block coefficient / hull form

## Heading Change vs Clearance

There is no strict IMO numerical standard, but empirical seamanship converges on:

| Phase | Typical Heading Change | Interpretation |
| --- | --- | --- |
| Initial kick | 10–20° | Stern starts moving off |
| Partial clearance | 30–45° | Propeller path diverging |
| Practical safety threshold | ~60° | Stern fully clear of original track |

Key insight:

👉 Clearance is not purely heading-based, but heading is used as a proxy for stern displacement.

# 3. Propeller Risk Envelope (Critical Safety Insight)

## Relative Motion Problem

The casualty is affected by:

- ship forward motion

- propeller suction / wash

- environmental drift (wind + current)

Thus, clearance requires:

D_{stern}(t) > D_{drift}(t) + R_{propeller}

Where:

- D_{stern}: lateral stern displacement

- D_{drift}: casualty drift

- R_{propeller}: propeller hazard radius

## Practical Seamanship Interpretation

A casualty can be considered clear when:

- Visually abaft beam and opening

- Outside wake turbulence

- Propeller wash no longer affecting body motion

- Confirmed by dedicated pointer (continuous bearing change)

# 4. Engine Use – Critical Trade-off

| Condition | Recommended Action |
| --- | --- |
| Immediate proximity (<1–2 ship lengths) | Engine → Neutral |
| Moderate distance + need for strong turn | Maintain propulsion briefly |
| After stern clear | Controlled propulsion |

Key trade-off:

- Propulsion ↑ → better turning moment

- Propulsion ↑ → higher propeller injury risk

# 5. Refined Professional Statement (Clean Version)

Here is your fully rewritten, publication-level formulation:

### Immediate Rudder–Propeller Response in MOB (Imminent Sight Scenario)

In an immediate Man Overboard (MOB) situation where the casualty is visually detected near the vessel, the primary objective is not recovery but instantaneous mitigation of propeller strike risk.

The standard seamanship practice is to apply maximum rudder toward the side of the casualty, thereby inducing a rapid yaw moment that shifts the vessel’s pivot point forward and generates a lateral stern swing away from the casualty. This hydrodynamic response, commonly referred to as the “kick effect,” develops within the first few seconds of rudder application and becomes operationally effective within approximately 10–20 seconds.

However, the critical unresolved question is the quantification of sufficient rudder application, specifically:

- the minimum duration and

- the required heading deviation

necessary to ensure that the propeller disc and stern trajectory are fully clear of the casualty’s position.

While traditional maneuvering guidance (e.g., Williamson or Anderson turns) suggests maintaining hard-over rudder up to 60° or more heading change, such values are primarily linked to recovery geometry rather than immediate propeller clearance. Therefore, a more precise safety criterion should be defined based on:

- stern lateral displacement (transfer),

- propeller wash influence zone,

- and relative drift of the casualty.

In practice, the casualty may be considered clear when:

- the stern has visibly diverged from the casualty’s bearing,

- propeller wash no longer affects the casualty,

- and the vessel has achieved sufficient lateral separation, typically approximated by a heading change in the order of 30–60°, depending on vessel characteristics.

# 6. Research Extension (Highly Relevant for Your Work)

Given your AIS-based modeling expertise, this can be formalized into:

### Proposed Model

- Extract turning trajectories from AIS

- Identify:

  - rate of turn (ROT)

  - transfer (lateral displacement)

- Simulate casualty as:

  - drifting particle (wind/current model)

👉 Output:

- Minimum safe rudder duration

- Probabilistic propeller risk envelope

# 7. Key Insight (Most Important)

There is no universal fixed standard for:

- “how long to keep hard rudder”

- “exact heading change for clearance”

Instead:

👉 It is a hydrodynamic + situational decision problem, typically approximated by:

- “hard over immediately, maintain until stern is visibly clear”

If you want, I can extend this into:

- a mathematical model for stern clearance time (τ_clear)

or a Python-based simulation using AIS trajectories + drift modeling (very suitable for your TRE paper). Yes. A useful way to define stern clearance time is as the earliest time at which the casualty is outside a conservative propeller hazard zone after the helm is put over.

I will build it in a form that is:

- physically interpretable,

- simple enough for simulation, and

- extendable to AIS-based calibration.

# 1. Objective

Define

\tau_{\text{clear}}

as the minimum time after MOB detection and helm action such that the casualty is no longer inside the stern–propeller danger region.

Formally,

\tau_{\text{clear}}=\inf\{t>0:\ d_{cp}(t)\ge R_h \}

where:

- d_{cp}(t) = distance between casualty and propeller centerline hazard reference at time t

- R_h = effective propeller hazard radius, including:

  - propeller radius

  - suction / wash influence

  - safety margin

  - uncertainty in drift and observation

# 2. Coordinate System

Use a body-fixed planar frame at t=0:

- origin at ship CG or midship

- x: forward

- y: starboard

- heading initially \psi(0)=0

Let the casualty enter water at:

\mathbf{r}_c(0)= \begin{bmatrix} x_{c0}\\ y_{c0} \end{bmatrix}

For an immediate MOB on the starboard beam, a simplified case is:

x_{c0}\approx 0,\qquad y_{c0}>0

Let the propeller hazard center initially be at stern location:

\mathbf{r}_p(0)= \begin{bmatrix} -x_s\\ 0 \end{bmatrix}

where x_s>0 is the longitudinal distance from CG to propeller plane.

# 3. Hazard Criterion

A casualty is considered clear when the distance from the casualty to the moving propeller hazard center exceeds the hazard radius:

\|\mathbf{r}_c(t)-\mathbf{r}_p(t)\| \ge R_h

Thus,

\tau_{\text{clear}}=\min\left\{t:\|\mathbf{r}_c(t)-\mathbf{r}_p(t)\|=R_h\right\}

for the first outward crossing.

# 4. Ship Motion Model During Initial Hard-Over “Kick”

For the first seconds after hard-over rudder, we only need the local stern swing, not the full recovery turn.

## 4.1 Heading response

A practical low-order yaw model is:

\dot{\psi}(t)=r(t)

\dot{r}(t)=\frac{K\delta-r(t)}{T_r}

where:

- r(t) = yaw rate

- \delta = rudder angle, usually hard-over

- K = steering gain

- T_r = yaw response time constant

If hard-over rudder is applied as a step input at t=0, then

r(t)=K\delta\left(1-e^{-t/T_r}\right)

and

\psi(t)=K\delta\left[t-T_r\left(1-e^{-t/T_r}\right)\right]

This is a good first analytical approximation.

## 4.2 Stern lateral displacement

Let the stern point be located a distance l_s aft of the pivot point. Then the stern transverse displacement caused by heading change is approximately

y_s(t)\approx -\,l_s \sin\psi(t)

For small to moderate heading change:

y_s(t)\approx -\,l_s\,\psi(t)

The negative sign means: if the bow turns to starboard, the stern moves to port.

This is the core of the kick effect.

## 4.3 Stern longitudinal shift

The stern also moves longitudinally:

x_s(t)\approx -\,l_s\cos\psi(t)

Relative to initial stern position, the longitudinal change is

\Delta x_s(t)=l_s\left(1-\cos\psi(t)\right)

For initial clearance, transverse separation usually dominates, but longitudinal shift can be retained in the full model.

# 5. Casualty Drift Model

For the first 10–30 seconds, casualty drift can be modeled as low-speed advection:

\mathbf{r}_c(t)=\mathbf{r}_c(0)+ \begin{bmatrix} u_c\\ v_c \end{bmatrix} t

where:

- u_c: longitudinal drift from current / ship-induced flow

- v_c: transverse drift from wind/current/body motion

A still-water first approximation is:

u_c\approx 0,\qquad v_c\approx 0

For more realism:

\begin{bmatrix} u_c\\ v_c \end{bmatrix} = \begin{bmatrix} u_{\text{curr}}\\ v_{\text{curr}} \end{bmatrix} + \begin{bmatrix} u_{\text{wash}}(t)\\ v_{\text{wash}}(t) \end{bmatrix}

# 6. Propeller Hazard Center Motion

Let the propeller hazard center move with stern swing. Then

\mathbf{r}_p(t)= \begin{bmatrix} x_{p0} + \Delta x_p(t)\\ y_{p0} + \Delta y_p(t) \end{bmatrix}

Using the pivot-point approximation:

\Delta x_p(t)=l_s\left(1-\cos\psi(t)\right)

\Delta y_p(t)=-\,l_s\sin\psi(t)

Thus

\mathbf{r}_p(t)= \begin{bmatrix} -x_s + l_s(1-\cos\psi(t))\\ -\,l_s\sin\psi(t) \end{bmatrix}

# 7. Full Clearance Equation

Now define relative position:

\Delta \mathbf{r}(t)=\mathbf{r}_c(t)-\mathbf{r}_p(t)

So

\Delta x(t)=x_{c0}+u_c t + x_s - l_s(1-\cos\psi(t))

\Delta y(t)=y_{c0}+v_c t + l_s\sin\psi(t)

The clearance condition becomes

\sqrt{\Delta x(t)^2+\Delta y(t)^2}=R_h

Therefore,

\tau_{\text{clear}}=\min\left\{t>0: \Delta x(t)^2+\Delta y(t)^2=R_h^2 \right\}

This is the most useful general definition.

# 8. Simplified Analytical Approximation

For the very early phase, let:

- casualty near beam: x_{c0}\approx 0

- negligible drift: u_c\approx v_c\approx 0

- small-to-moderate heading change: \sin\psi\approx\psi,\ \cos\psi\approx 1

Then

\Delta x(t)\approx x_s

\Delta y(t)\approx y_{c0}+l_s\psi(t)

Hence

d_{cp}^2(t)\approx x_s^2+\left(y_{c0}+l_s\psi(t)\right)^2

So clearance occurs when

x_s^2+\left(y_{c0}+l_s\psi(\tau_{\text{clear}})\right)^2=R_h^2

If R_h^2>x_s^2, then

y_{c0}+l_s\psi(\tau_{\text{clear}}) = \sqrt{R_h^2-x_s^2}

thus

\psi(\tau_{\text{clear}}) = \frac{\sqrt{R_h^2-x_s^2}-y_{c0}}{l_s}

Now substitute the heading response:

K\delta\left[\tau_{\text{clear}}-T_r\left(1-e^{-\tau_{\text{clear}}/T_r}\right)\right] = \frac{\sqrt{R_h^2-x_s^2}-y_{c0}}{l_s}

This is an implicit analytical equation for stern clearance time.

# 9. Very Simple Closed-Form Approximation

If t\ll T_r, then from the exponential expansion:

1-e^{-t/T_r}\approx \frac{t}{T_r}-\frac{t^2}{2T_r^2}

so

\psi(t)\approx \frac{K\delta}{2T_r}t^2

Then

\frac{K\delta}{2T_r}\tau_{\text{clear}}^2 = \frac{\sqrt{R_h^2-x_s^2}-y_{c0}}{l_s}

and

\boxed{ \tau_{\text{clear}} \approx \sqrt{ \frac{2T_r}{K\delta\, l_s} \left(\sqrt{R_h^2-x_s^2}-y_{c0}\right) } }

This is the cleanest compact expression for early-time stern clearance.

# 10. Interpretation of Parameters

## K\delta

Higher effective steering moment reduces \tau_{\text{clear}}.

- larger rudder angle

- higher inflow speed over rudder

- better steering effectiveness

## T_r

Larger yaw lag increases \tau_{\text{clear}}.

- sluggish ship

- delayed turning response

- loaded tanker condition

## l_s

Greater pivot-to-stern arm improves the kick effect.

- if pivot point is far forward, stern moves more for same heading change

## y_{c0}

If casualty falls farther to the side, initial danger may actually be lower.

## R_h

Larger assumed hazard radius increases required clearance time.

This is important because the true danger zone is not just the propeller disc. It should include:

R_h = R_p + R_{\text{wash}} + R_{\text{obs}} + R_{\text{margin}}

# 11. Recommended Practical Hazard Radius Formulation

For modeling, define:

R_h = \alpha D_p + \beta V_s

or more explicitly,

R_h = \frac{D_p}{2} + R_{\text{wash}} + R_{\text{uncert}}

where:

- D_p: propeller diameter

- R_{\text{wash}}: wash/suction radius

- R_{\text{uncert}}: casualty location uncertainty

For a conservative operational model, you may treat

R_h \in [1.5D_p,\ 3D_p]

depending on safety philosophy and vessel type.

# 12. AIS/Ship-Handling Calibration Path

This model becomes publishable when the abstract parameters are estimated from real maneuvering data.

## Estimate from sea trial / AIS / pilot data:

- K, T_r: from initial ROT response after helm command

- l_s: from pivot point estimate

- u_c, v_c: from current/wind assumptions

- R_h: from safety criterion

You can fit yaw response from observed ROT:

r(t)=r_\infty(1-e^{-t/T_r})

where

r_\infty=K\delta

Then integrate for heading, then compute stern path, then solve for \tau_{\text{clear}}.

# 13. Recommended Final Definition for a Paper

You can write it like this:

The stern clearance time, \tau_{\text{clear}}, is defined as the minimum elapsed time after hard-over rudder application at which the casualty exits the effective propeller hazard zone. The hazard zone is modeled as a moving circular region centered at the propeller plane, with radius R_h, accounting for propeller geometry, hydrodynamic suction/wash, and operational uncertainty. Thus,

> \tau_{\text{clear}}=\inf\{t>0:\|\mathbf{r}_c(t)-\mathbf{r}_p(t)\|\ge R_h\} >

where \mathbf{r}_c(t) is the casualty position and \mathbf{r}_p(t) is the propeller hazard center trajectory induced by the vessel’s initial yaw response. The latter is driven by a first-order steering dynamics model, enabling stern clearance to be expressed as a function of rudder angle, yaw response time constant, pivot-point geometry, and casualty drift.

# 14. Compact Final Model Summary

## General model

\tau_{\text{clear}}=\min\left\{t>0: \left[x_{c0}+u_ct+x_s-l_s(1-\cos\psi(t))\right]^2+ \left[y_{c0}+v_ct+l_s\sin\psi(t)\right]^2 \ge R_h^2 \right\}

with

\psi(t)=K\delta\left[t-T_r\left(1-e^{-t/T_r}\right)\right]

## Early-time approximation

\psi(t)\approx \frac{K\delta}{2T_r}t^2

\boxed{ \tau_{\text{clear}} \approx \sqrt{ \frac{2T_r}{K\delta\, l_s} \left(\sqrt{R_h^2-x_s^2}-y_{c0}\right) } }

# 15. Important Limitation

This model captures the first protection phase, not the full MOB recovery maneuver.

It does not yet include:

- crash stop effects

- propeller reverse thrust

- shallow water bank effects

- wave-induced body motion

- twin-screw asymmetry

- nonuniform propeller wash field

But as a first paper-grade model, it is strong, interpretable, and expandable.

Next, I can turn this into either a diagram + notation table, or a Python function that computes \tau_{\text{clear}} from vessel parameters.

Short answer: this can stand as a fully independent study, but whether you should separate it depends on your publication strategy and how you position the contribution.

Below is a structured, research-oriented evaluation.

# 1. Conceptual Positioning

## Your current project (inferred)

- Focus: Dynamic pilot boarding / constrained water operations

- Core method: AIS + environmental constraints + optimization

- Objective: operational efficiency + safety (collision, grounding)

## New idea (this work)

- Focus: Immediate MOB protection (first 10–30 seconds)

- Core method: maneuvering dynamics + safety envelope modeling

- Objective: propeller strike risk minimization

👉 These are fundamentally different problem classes:

| Dimension | Pilot Boarding Study | MOB Clearance Study |
| --- | --- | --- |
| Time scale | Minutes–hours | Seconds |
| Decision type | Strategic / operational | Immediate / reactive |
| Modeling | Traffic + environment + optimization | Hydrodynamics + control response |
| Output | Optimal location / relocation | Safety time threshold (τ_clear) |
| Risk type | Collision / grounding | Human injury (propeller) |

Conclusion:

→ Scientifically, this is not a sub-module; it is a distinct problem domain.

# 2. Novelty Assessment (Important for Publication)

## What exists in literature

Typical MOB literature:

- Procedural (Williamson / Anderson turn)

- Qualitative seamanship guidance

- No quantitative propeller-risk modeling

## Your contribution

You introduce:

- Formal definition of τ_clear

- Dynamic propeller hazard envelope

- Coupling of yaw response + human drift

- Potential AIS-based calibration

👉 This is novel in maritime safety research.

You are essentially moving from:

“Turn hard over” (procedural)

to

“Minimum time-to-propeller-clearance as a function of vessel dynamics” (quantitative)

That is publishable as a standalone contribution.

# 3. Two Strategic Options

## OPTION A — Independent Paper (Recommended for impact)

### Title direction

- “Time-to-Propeller Clearance in Immediate MOB Scenarios: A Dynamic Maneuvering Model”

- “Quantifying Stern Clearance Time for Man Overboard Safety”

### Contribution

- Define τ_clear

- Derive model (as you did)

- Validate with:

  - maneuvering parameters

  - simulated scenarios

- Possibly extend:

  - sensitivity analysis (speed, rudder angle, vessel type)

### Target journals

- Ocean Engineering

- Applied Ocean Research

- Reliability Engineering & System Safety

- Safety Science

👉 Strong theoretical + applied paper.

## OPTION B — Sub-study within your current project

This only works if you frame it as:

“Emergency safety constraint within pilot boarding / constrained water operations”

Then:

- τ_clear becomes a constraint function

- You integrate it into:

  - meeting point optimization

  - traffic safety modeling

Example:

\text{Safe zone} = f(\text{UKC, traffic, environment, } \tau_{\text{clear}})

### Risk of this approach

- Your contribution gets diluted

- Reviewers may miss the novelty

- Paper becomes too broad (common rejection reason in TRE-type journals)

# 4. Recommended Strategy (Academic + Practical)

## Best approach:

## Two-paper architecture

### Paper 1 (this study — core innovation)

👉 MOB dynamic safety model

- Develop τ_clear

- Provide analytical + simulation framework

- Possibly classify vessel types (tanker, container, etc.)

### Paper 2 (your existing project)

👉 Pilot boarding optimization

- Use τ_clear as:

  - safety constraint

  - validation metric

Example integration:

\text{Feasible meeting point} \Rightarrow \tau_{\text{clear}} < \tau_{\text{threshold}}

This creates a strong methodological linkage without sacrificing novelty.

# 5. Why Separation is Stronger (Reviewer Perspective)

Reviewers typically look for:

- Clear problem definition

- Single dominant contribution

- Minimal conceptual diffusion

If combined:

❌ “This paper studies pilot boarding, AIS traffic, and MOB dynamics”

→ too broad

If separated:

✅ Paper 1: “We solve a previously unquantified safety problem”

✅ Paper 2: “We apply that safety model in a real operational system”

# 6. Research Value (Your Advantage)

This direction aligns extremely well with your expertise:

- You already work with AIS + trajectory modeling

- You understand real maneuvering behavior

- You can bridge:

  - theory ↔ real operations

👉 This gives you a rare advantage:

Most MOB studies are written by:

- safety officers (procedural)

- not by data-driven maneuvering analysts

# 7. Potential Extensions (High Impact)

If developed independently, you can extend to:

## A. Probabilistic risk model

P(\text{propeller strike}) = f(\tau_{\text{response}}, V, \delta, drift)

## B. Vessel classification

- Tanker vs container vs LNG

- Compare τ_clear distributions

## C. Real AIS validation

- Detect near-MOB-like sudden maneuvers

- Estimate real turning response

## D. Bridge decision support

- “Minimum action time before irreversible risk”

# 8. Final Recommendation

👉 Make this a separate study (primary paper)

👉 Then integrate it into your pilot boarding project (secondary paper)

# 9. One-Line Positioning (Use This)

You can frame your research like this:

“This study introduces a quantitative model for stern clearance time in immediate MOB scenarios, transforming traditional procedural seamanship into a measurable safety criterion based on vessel maneuvering dynamics.”

If you want next step, I can:

- design paper structure (abstract → methodology → results)

- or build a Python simulation framework for τ_clear using realistic tanker parameters (very suitable for your TRE pipeline)